ReadMe for cryo log data:

The data in each of the logs are organised into columns and should be opened with Microsoft Excel (tested on Excel 2010 edition).

The data shows the parameters of the cryo system chronologically and the file name relates to the date the data was recorded through the LabVIEW Virtual Instrument (VI) and operated on LabVIEW 2013 SP1. The columns relating to the data are as follows:

Time: The time at which the line of data was recorded. If a data log spans for longer than one day an advance in the date by one day should be noted as the VI does not account for this. If the VI is stopped and restarted there will be a line-break in the time field stating “##### New log start #####”.

Counter: Time in milliseconds since the VI has been started, as the sampling rate can be adjusted in the VI.

Setpoint Temp: The temperature to which the temperature controller is regulating the system, measured in Kelvin.

Temp_(A,B,C,D): The current temperature of the temperature sensors connected to input A, B, C and D respectively measured in Kelvin.

Setpoint Pressure: The pressure to which the gas system is regulating, measured in mbar.

Press_1: The pressure inside the growth chamber, measured in mbar.

Press_2: The pressure in the reservoir of the gas system, measured in mbar.

Press_Chamber: The vacuum chamber pressure, measured in mbar.

VolumeCup: The gas volume in the growth chamber, measured in standard cubic centimeters. This is the difference in the volume of gas into the growth chamber and that which has been pumped out.

Comment: Comments made at the timestamp. The comments persist on each loop of the VI until the comment has been changed, so most comments can be discounted after their first instance.